# R V College of Engineering

# Computer Science and Engineering

### Field Visit’s to Kolar, Karnataka

### Real-Time Bat Acoustic Monitoring with ML Classification

In collaboration with Bat Conservation India Trust (BCIT) and Centre for Wildlife Studies (CWS)

Faculty Advisor:

Dr. Anala M R, Professor (ISE Dept.)

Student Coordinator:

Nihal Soni (1RV23CS152)
 Nikhil Parashuram Bakale (1RV23CS152)

## Brief Overview

As part of our ongoing research initiative, we designed and developed a Real-Time Bat Acoustic Monitoring System integrating ultrasonic microphones, environmental sensors, Raspberry Pi, and long-range LoRa communication.
The project aims to detect, record, process, and classify bat ultrasonic calls using ML-based spectrogram analysis.

To validate the system’s performance in real-world wildlife environments, multiple field tests were conducted at various locations in Kolar, Karnataka, including clear LOS, obstructed areas, and cave-like environments with no internet connectivity.

## Objectives of Field Testing

- Validate client–server communication reliability in outdoor conditions

- Test repeater node behavior

- Evaluate environmental sensor response in natural settings

- Verify system performance in both line-of-sight and non-line-of-sight environments

- Assess communication in non-networked/no-internet zones

- Test two prototype versions:

  - SX1278-based LoRa module

  - EBYTE UART-LoRa module

- Identify system limitations and gather insights for improvement

## Summary of Activities Performed

### ✔ Communication & Synchronization Tests

- Validated reliable client–server LoRa communication

- Tested repeater functionality for range extension

- Verified flow control system, including feedback and pacing

- Confirmed missing packet detection, error correction, and automatic retransmission

- Ensured proper synchronization between transmitting and receiving nodes

- Successfully validated ACK/NACK acknowledgment system

- Tested all possible edge cases (packet loss, interference, packet disorder)

### ✔ Range & Environmental Testing

- Performed long-distance range testing across multiple terrains

- Tested communication in clear LOS and NLOS areas, including closed and semi-enclosed spaces

- Verified reliability in no-internet conditions, ensuring full offline operability

- Observed LoRa signal behavior under different physical obstructions

### ✔ Hardware, Sensors & Acoustic Testing

- Evaluated ultrasonic microphone performance in natural bat habitat conditions

- Tested environmental sensors for temperature, humidity, pressure, and light data consistency

- Compared performance between two prototype versions (SX1278 and EBYTE LoRa)

### ✔ System & Application Testing

- Conducted full test of real-time web dashboard monitoring

- Tested remote triggering of client and server functions via admin interface

- Evaluated admin dashboard features

- Collected insights for communication stability, casing design, and antenna placement

## Outcomes and Learnings

### Technical Outcomes

- Validated long-range LoRa communication in diverse outdoor conditions

- Improved understanding of packet-level behavior, synchronization, and retransmission logic

- Gained clarity on hardware design improvements for performance and stability

- Identified practical field limitations for future optimization

### Student Learnings

- Hands-on experience in IoT deployment in real environmental conditions

- Understanding challenges of working in non-networked and low-connectivity environments

- Strengthened skills in hardware debugging and wireless communication

- Developed efficient teamwork, planning ability, and on-ground coordination experience

## Concluding Remarks

The field testing conducted in Kolar significantly contributed to validating the Real-Time Bat Acoustic Monitoring System developed under the BCIT–CWS collaboration.
The tests ensured the reliability of LoRa communication, sensor accuracy, acoustic monitoring performance, and administrator-side system controls, providing valuable technical insights for further development.

# Student Participation & Eligibility

| SL No. | NAME | USN | Activity Points |
| --- | --- | --- | --- |
| 1. | Nihal Soni | 1RV23CS152 | 40 |
| 2. | Nikhil Parashuram Bakale | 1RV23CS154 | 40 |

Both students have successfully completed Three Field Visits associated with this project and are eligible for 40 AICTE Activity Points.

Pictures of Field Visits: